PTZ2 Specifications:

- Zoom Range: 10x Optical Only

- Pan range: +/-100

- Tilt range: +15/-30

- Type: 1/3" Hi-res color CCD

- Resolution: 420 lines

- NTSC video format

- Max horiz field of view: 66deg with internal lens

- Removable Sony VCL-0637A .6X wide-angle lens 37mm screw mount (inside camera head)

- Min illumination: 7 lux

- Composite vieo output on camera base

- Y/C or "S-Video" output available through minidin8 using CABPTELPLUS.

- Auto focus: 6" to infinity

- Automatic and manual exposure and white balance control

- Pan and tilt rate proportional to zoom setting

- Fast zooming during “preset” operation

- Simultaneous Pan, Tilt and Zoom operation

- Combined Pan, Tilt & Zoom presets in 2 seconds max

- Standard tripod mount hole on bottom

- PictureTel 8-pin camera interface (pwr, comm, Y/C video)

- DC power jack for optional remote power (2.5 mm tip neg)

- 12v 1.0A power (1.5A peak) for PC compatibility

- Fast, quiet and reliable pan tilt mechanism

- Privacy Mode - Camera faces back and turns off power

- Infrared receiver and decoder for wireless control

- RS- 232 (EIA/TIA - 232E) serial communication port

- Software, with the exception of the Video Head SW, is completely downloadable through RS-232 port

- Software saved in non-volatile memory (survives power down)

- Mates with the optional Limelight base to add auto pointing based on speech

Pan Range:
Design Specification: Minimum of ±100o from forward position at any tilt angle, in horizontal plane
Production Specification: +/- 100o ± 0.5o  

Tilt Range:
Design Specification: Minimum +15/-30 o over ±100 o pan range
Production Specification: + 15o ± 0.5o,  - 30o ± 0.5o

Pan/Tilt manual speed (jogging speed):
Manual Pan/tilt speed is variable based on the camera’s field of vision (zoom position). The camera supports variable “speed limit control” modes selected via system serial commands. Camera speed is measured by timing the rate that a full scene displayed on a standard (underscanned) NTSC monitor changes from one end of the display to the other (seconds per screen or “sps”). All camera manual speed ratings are accurate within ±10%. The rates for various zoom positions are as follows.
Pan speed at full wide: 3.25 sps. (20.3 o /s)
Pan speed at full tele: 1.50 sps. (3.6 o /s)
Tilt speed at full wide: 2.45 sps. (20.2 o /s)
Tilt speed at full tele: 1.30 sps. (3.12 o /s) 

Pan/Tilt preset speed:
Maximum 2 seconds for any combination of pan, tilt and zoom.

Positioning accuracy:
Design Specification: ±0.5o absolute position accuracy per axis (pan or tilt).
Production Specification: ±1.0o absolute position accuracy per axis (pan or tilt).

Preset position repeatability:
Design Specification: ±0.2o position repeatability, pan and tilt
Production Specification: ±0.5o position repeatability, pan and tilt